---------------------------------------------------------------------------------------------------------------------

The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

A high resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/uv16med-fd.

---------------------------------------------------------------------------------------------------------------------

FOR IMMEDIATE RELEASE

UV Curable System with a Fluorescent Dye Meets ISO 10993-5 Testing

Master Bond UV16Med-FD is a one part, low viscosity, epoxy based UV curable system containing a fluorescent dye for identification. This feature enables visual inspection with a UV black light to detect voids or overflow and confirm product placement. Engineered for bonding, sealing, coating, and spin coating across a variety of medical device applications, this system passes ISO 10993-5 standards for non-cytotoxicity.

UV16Med-FD resists acids, bases, solvents, and sterilants including ethylene oxide and glutaraldehyde. It also withstands repeated autoclaving. The cured material is optically clear, with a refractive index of 1.511 at 75°F. Tensile strength measures 7,000–8,000 psi, and tensile modulus ranges from 250,000–300,000 psi at room temperature. A low viscosity of 250–500 cps at 75°F makes the material suitable for application by syringe or spin coating. It adheres to substrates including plastics, glass, ceramics, and metals.

UV16Med-FD is a cationic system that contains no solvents or diluents and is not oxygen inhibited. Exposure to UV light at 320–365 nm, with a minimum intensity of 20–40 mW/cm², initiates the cure. The cure rate depends on light intensity, distance from the source, and section thickness. Post curing at 125°C for 30 minutes brings the glass transition temperature to 130–135°C, which improves autoclave resistance. The service temperature range spans -80°F to +400°F (-62°C to +204°C). UV16Med-FD is available in 1/2 pint, pint, quart, gallon, and 5 gallon containers, as well as EFD® syringes compatible with many automated dispensers.

Master Bond Biocompatible Adhesives

Master Bond manufactures adhesives, sealants, coatings, and potting/encapsulation compounds tested to USP Class VI and/or ISO 10993-5 standards for use in medical device applications. Available chemistries include epoxies, silicones, UV/LED curable systems, and dual cure formulations. Learn more about Master Bond’s biocompatible systems at masterbond.com/properties/biocompatible-adhesives or contact our technical support team to discuss your specific application requirements.

TECH SUPPORT

Email: technical@masterbond.com

Web: https://www.masterbond.com/contact

Tel: +1-201-343-8983

Note to Editors:

For a full product description, please visit https://www.masterbond.com/tds/uv16med-fd

Check out new videos on our YouTube channel: https://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com